SLR Parser Construction

- 과목명:		컴파일러

- 담당교수님: 	문희승 교수님

- 학과: 		소프트웨어학부

- 그룹: 			   1032

- 보고서작성자: 20220702 서정민

[목차]

1. Grammar Refactoring

1-1. dangling else 문제

1-2. 연산자 우선순위와 결합 순서 문제

2. SLR(1) Table Construction

2-1. Table Construction

2-2. Follow 집합이 테이블의 충돌을 해결하는 예시

3. Parser Implementation

3-1. 구현

3-2. 결과 예시

1. Grammar Refactoring

주어진 문법에서 문법이 모호한 문제를 먼저 해결한다.

1-1. 11, 12번의 dangling else 문제

11. Stmt -> if ( Expr ) Stmt

12. Stmt -> if ( Expr ) Stmt else Stmt

이 문법은 모호하다. 11번에서 18번까지의 문법을 아래와 같이 수정한다.

Stmt -> IfStmt | OtherStmt

IfStmt -> matched_Stmt | open_Stmt

matched_Stmt -> if ( Expr ) then matched_Stmt else matched_Stmt | OtherStmt

open_Stmt -> if ( Expr ) then Stmt | if ( Expr ) then matched_Stmt else open_Stmt

OtherStmt -> while ( Expr ) Stmt | for ( Expr ; Expr ; Expr ) Stmt | return Expr; | VarDecl | ExprStmt | Block

1-2. 20번에서 27번까지의 연산자 우선순위와 결합 순서 문제

20. Expr -> Expr == Expr

21. Expr -> Expr + Expr

22. Expr -> Expr * Expr

이 문법은 모호하다. 연산자 우선순위 문제가 있고, 그 아래도 우선순위 문제로 수정해야 한다.

Expr -> EqualExpr

EqualExpr -> EqualExpr == AddExpr | AddExpr

AddExpr -> AddExpr + MulExpr | MulExpr

MulExpr -> MulExpr * UnaryExpr | UnaryExpr

UnaryExpr -> - UnaryExpr | OtherExpr

OtherExpr -> id ( ArgList ) | id | num | ( Expr )

수정한 전체 문법은 아래와 같다.

- Program -> DeclList

- DeclList -> Decl DeclList | epsilon

- Decl -> VarDecl | FuncDecl

- VarDecl -> type id ; | type id = Expr ;

- FuncDecl -> type id ( ParamList ) Block

- ParamList -> Param, ParamList | Param | epsilon

- Param -> type id

- Block -> { StmtList }

- StmtList -> Stmt StmtList | epsilon

- Stmt -> IfStmt | OtherStmt

- IfStmt -> matched_Stmt | open_Stmt

- matched_Stmt -> if ( Expr ) then matched_Stmt else matched_Stmt | OtherStmt

- open_Stmt -> if ( Expr ) then Stmt | if ( Expr ) then matched_Stmt else open_Stmt

- OtherStmt -> while ( Expr ) Stmt | for ( Expr ; Expr ; Expr ) Stmt | return Expr; | VarDecl | ExprStmt | Block

- ExprStmt -> id = Expr ;

- Expr -> EqualExpr

- EqualExpr -> EqualExpr == AddExpr | AddExpr

- AddExpr -> AddExpr + MulExpr | MulExpr

- MulExpr -> MulExpr * UnaryExpr | UnaryExpr

- UnaryExpr -> - UnaryExpr | OtherExpr

- OtherExpr -> id ( ArgList ) | id | num | ( Expr )

- ArgList -> Expr, ArgList | Expr | epsilon

두번째로 reduce/reduce 충돌을 해결한다.

Program -> DeclList

DeclList -> Decl DeclList

DeclList -> epsilon

Decl -> VarDecl

Decl -> FuncDecl

VarDecl -> type id ;

VarDecl -> type id = Expr ;

FuncDecl -> type id ( ParamList ) Block

ParamList -> Param , ParamList

ParamList -> Param

ParamList -> epsilon

Param -> type id

Block -> { StmtList }

StmtList -> Stmt StmtList

StmtList -> epsilon

Stmt -> IfStmt

Stmt -> OtherStmt

IfStmt -> matched_Stmt

IfStmt -> open_Stmt

matched_Stmt -> if ( Expr ) then matched_Stmt else matched_Stmt

matched_Stmt -> OtherStmt

open_Stmt -> if ( Expr ) then Stmt

open_Stmt -> if ( Expr ) then matched_Stmt else open_Stmt

OtherStmt -> while ( Expr ) Stmt

OtherStmt -> for ( Expr ; Expr ; Expr ) Stmt

OtherStmt -> return Expr ;

OtherStmt -> VarDecl

OtherStmt -> ExprStmt

OtherStmt -> Block

ExprStmt -> id = Expr ;

Expr -> EqualExpr

EqualExpr -> EqualExpr == AddExpr

EqualExpr -> AddExp

AddExpr -> AddExpr + MulExpr

AddExpr -> MulExpr

MulExpr -> MulExpr * UnaryExpr

MulExpr -> UnaryExp

UnaryExpr -> - UnaryExpr

UnaryExpr -> OtherExpr

OtherExpr -> id ( ArgList )

OtherExpr -> id

OtherExpr -> num

OtherExpr -> ( Expr )

ArgList -> Expr , ArgList

ArgList -> Expr

ArgList -> epsilon

이 문법을 이용해서 SLR Table Construction을 진행한다.

https://jsmachines.sourceforge.net/machines/slr.html 과제에서 주어진 이 사이트를 이용한다. 위의 문법으로 바로 Table Construction을 해보면 충돌이 발생하는 것을 확인할 수 있다.

위 문법에서 구한 FIRST, FOLLOW set

위 문법에서 구한 LR Table

먼저, 윗부분에서 Reduce/reduce 충돌이 발생하고 있다. 이것은 위의 수정한 문법에서 10번이 불필요하기 때문이다.

10. Stmt -> IfStmt | OtherStmt

Stmt -> OtherStmt

Matched_Stmt -> OhterStmt

두개의 축약 규칙이 동시에 적용되기에 발생하는 문제이다.

다시 문법을 수정한다.

- Program -> DeclList

- DeclList -> Decl DeclList | epsilon

- Decl -> VarDecl | FuncDecl

- VarDecl -> type id ; | type id = Expr ;

- FuncDecl -> type id ( ParamList ) Block

- ParamList -> Param, ParamList | Param | epsilon

- Param -> type id

- Block -> { StmtList }

- StmtList -> Stmt StmtList | epsilon

- Stmt -> matched_Stmt | open_Stmt

- matched_Stmt -> if ( Expr ) then matched_Stmt else matched_Stmt | OtherStmt

- open_Stmt -> if ( Expr ) then Stmt | if ( Expr ) then matched_Stmt else open_Stmt

- OtherStmt -> while ( Expr ) Stmt | for ( Expr ; Expr ; Expr ) Stmt | return Expr; | VarDecl | ExprStmt | Block

- ExprStmt -> id = Expr ;

- Expr -> EqualExpr

- EqualExpr -> EqualExpr == AddExpr | AddExpr

- AddExpr -> AddExpr + MulExpr | MulExpr

- MulExpr -> MulExpr * UnaryExpr | UnaryExpr

- UnaryExpr -> - UnaryExpr | OtherExpr

- OtherExpr -> id ( ArgList ) | id | num | ( Expr )

- ArgList -> Expr, ArgList | Expr | epsilon

OtherStmt가 한 곳에서만 축약되도록 문법을 수정 후 다시 Table Construction한다.

Program -> DeclList

DeclList -> Decl DeclList

DeclList -> epsilon

Decl -> VarDecl

Decl -> FuncDecl

VarDecl -> type id ;

VarDecl -> type id = Expr ;

FuncDecl -> type id ( ParamList ) Block

ParamList -> Param , ParamList

ParamList -> Param

ParamList -> epsilon

Param -> type id

Block -> { StmtList }

StmtList -> Stmt StmtList

StmtList -> epsilon

Stmt -> matched_Stmt

Stmt -> open_Stmt

matched_Stmt -> if ( Expr ) then matched_Stmt else matched_Stmt

matched_Stmt -> OtherStmt

open_Stmt -> if ( Expr ) then Stmt

open_Stmt -> if ( Expr ) then matched_Stmt else open_Stmt

OtherStmt -> while ( Expr ) Stmt

OtherStmt -> for ( Expr ; Expr ; Expr ) Stmt

OtherStmt -> return Expr ;

OtherStmt -> VarDecl

OtherStmt -> ExprStmt

OtherStmt -> Block

ExprStmt -> id = Expr ;

Expr -> EqualExpr

EqualExpr -> EqualExpr == AddExpr

EqualExpr -> AddExpr

AddExpr -> AddExpr + MulExpr

AddExpr -> MulExpr

MulExpr -> MulExpr * UnaryExpr

MulExpr -> UnaryExpr

UnaryExpr -> - UnaryExpr

UnaryExpr -> OtherExpr

OtherExpr -> id ( ArgList )

OtherExpr -> id

OtherExpr -> num

OtherExpr -> ( Expr )

ArgList -> Expr , ArgList

ArgList -> Expr

ArgList -> epsilon

reduce/reduce 충돌이 해결되었다. 그러나 아래 shift/reduce 충돌이 있다.

IfStmt 에서 ElseStmt 부분을 분리하고 if 문 안에서 Stmt 자리에 Block을 사용해서 끝을 명확하게 하도록 만들었다. 그리고 SLR(1) 테이블 생성 이전에 Program' -> Program 부분을 추가하고 테이블을 생성한다.

Program' -> Program

Program -> DeclList

DeclList -> Decl DeclList

DeclList -> epsilon

Decl -> VarDecl

Decl -> FuncDecl

VarDecl -> type id ;

VarDecl -> type id = Expr ;

FuncDecl -> type id ( ParamList ) Block

ParamList -> Param , ParamList

ParamList -> Param

ParamList -> epsilon

Param -> type id

Block -> { StmtList }

StmtList -> Stmt StmtList

StmtList -> epsilon

Stmt -> IfStmt

Stmt -> OtherStmt

IfStmt -> if ( Expr ) then Block ElseStmt

ElseStmt -> else Block

ElseStmt -> epsilon

OtherStmt -> while ( Expr ) Stmt

OtherStmt -> for ( Expr ; Expr ; Expr ) Stmt

OtherStmt -> return Expr ;

OtherStmt -> VarDecl

OtherStmt -> ExprStmt

OtherStmt -> Block

ExprStmt -> id = Expr ;

Expr -> EqualExpr

EqualExpr -> EqualExpr == AddExpr

EqualExpr -> AddExpr

AddExpr -> AddExpr + MulExpr

AddExpr -> MulExpr

MulExpr -> MulExpr * UnaryExpr

MulExpr -> UnaryExpr

UnaryExpr -> - UnaryExpr

UnaryExpr -> OtherExpr

OtherExpr -> id ( ArgList )

OtherExpr -> id

OtherExpr -> num

OtherExpr -> ( Expr )

ArgList -> Expr , ArgList

ArgList -> Expr

ArgList -> epsilon

2. SLR Table Construction

2-1. Table Construction

이전에 발생한 충돌이 해결된 것을 확인할 수 있다.

2-2. Follow 집합이 테이블의 충돌을 해결하는 예시

IfStmt   -> if ( Expr ) then Block ElseStmt

ElseStmt -> else Block

ElseStmt -> epsilon

문법 중 이러한 부분이 있다.

IfStmt -> if ( Expr ) then Block  ·  ElseStmt

여기까지 읽은 상태에서

ElseStmt ->  ·  else Block (shift)

ElseStmt ->  ·  epsilon (reduce)

Shift/reduce 충돌이 발생한다

이때 Follow(IfStmt) = Follow(ElseStmt) = { epsilon, if, while, for, return, type, {, id }

Lookahead가 else일때, else는 Follow(ElseStmt)에 들어있지 않으므로 reduce(epsilon) 이 아닌 shift(else) 한다. 만약 lookahead가 Follow(ElseStmt)에 속하는 다른 토큰이라면 reduce 한다.

이러한 방법으로 if ( Expr) then Block ElseStmt 형태의 상황에서 else를 shift 해서 else 구문까지 처리하고, if ( Expr ) then Block의 else가 없는 상황에서는 reduce(epsilon) 되어서 else 없는 if구문을 처리한다.

3. Parser Implementation

3-1. 구현

from typing import List, Tuple, Union

from collections import defaultdict

import re

class ParseTree:

def __init__(self, symbol, children=None, value=None):

self.symbol = symbol

self.children = children if children is not None else []

self.value = value

def __str__(self):

return self._to_string()

def _to_string(self, level=0):

indent = "  " * level

result = f"{indent}{self.symbol}"

if self.value is not None and self.value != self.symbol:

result += f": {self.value}"

result += "\n"

for child in self.children:

result += child._to_string(level + 1)

return result

class ErrorReport:

def __init__(self, position: int, message: str):

self.position = position

self.message = message

def parser(tokens: List[str]) -> Tuple[bool, Union[ParseTree, ErrorReport]]:

"""

Returns (True, parse_tree) on success or (False, ErrorReport) on failure.

Example:

return True, ParseTree(...)

return False, ErrorReport(3, "unexpected token 'else'")

"""

SLR_TABLE = make_table(given_grammar)

tokens = preprocess_read_tokens(tokens)

# 초기상태와 EOF 붙이기

stack = [0]

tokens += ["$"]

index = 0

while True:

if len(stack) == 0:

return False, ErrorReport(index, "스택이 비어있습니다")

else:

current_state = stack[-1]

action = SLR_TABLE.get(stack[-1], {}).get(tokens[index])

if action == "acc":  # accept 경우

return True, stack[1]

elif action is None:  # 테이블 결과가 없는 경우

expected = list(SLR_TABLE.get(stack[-1], {}).keys())

return False, ErrorReport(

index,

f"unexpected token '{tokens[index]}', expected one of {expected}"

)

elif isinstance(action, str):

if action[0] == "s":  # shift 경우

terminal = ParseTree(tokens[index])

stack.append(terminal)

stack.append(int(action[1:]))

index += 1

elif action[0] == "r":  # reduce 경우

rule_index = int(action[1:])

lhs, rhs = given_grammar[rule_index]

children = []

if rhs == ["epsilon"]:

pass

else:

for _ in range(len(rhs)):

if len(stack) < 1:  # 상태 pop

return False, ErrorReport(

index,

f"reduce 위한 상태 pop 이전에 스택 언더플로우 발생. Rule: {lhs} -> {' '.join(rhs)}",

)

stack.pop()

if len(stack) < 1:  # 노드 pop

return False, ErrorReport(

index,

f"reduce 위한 노드 pop 이전에 스택 언더플로우 발생. Rule: {lhs} -> {' '.join(rhs)}",

)

node_to_add = stack.pop()  # ParseTree 노드 pop

if not isinstance(node_to_add, ParseTree):

return False, ErrorReport(

index,

f"스택의 parseTree위치에 Type이 존재함, got {type(node_to_add)}. Rule: {lhs} -> {' '.join(rhs)}",

)

children.insert(0, node_to_add)

parent_node = ParseTree(lhs, children)

# goto

prev_state = stack[-1]

if not isinstance(prev_state, int):

return False, ErrorReport(

index,

f"Invalid state {prev_state}, 축소후 스택 top에 int가 아닌 값이 있음",

)

goto = SLR_TABLE.get(prev_state, {}).get(lhs)

if goto is None:

return False, ErrorReport(

index, f"상태 {prev_state} 에서 심볼 {lhs}에 대한 goto가 정의되어 있지 않습니다."

)

if not isinstance(goto, int):

return False, ErrorReport(

index, f"현재 goto값 {goto}는 정수가 아님."

)

stack.append(parent_node)

stack.append(goto)

return False, ErrorReport(index, "SLR parser loop 비정상 종료")

def make_table(raw_grammar):

# ε 처리 & 인덱스 매핑

grammar = []

for lhs, rhs in raw_grammar:

grammar.append((lhs, [] if rhs == ["epsilon"] else rhs))

nonterminals = {lhs for lhs, _ in grammar}

terminals = set(

sym for _, rhs in grammar for sym in rhs

if sym not in nonterminals

) | {"$"}

# FIRST

FIRST = {X: set() for X in nonterminals}

for t in terminals:

FIRST[t] = {t}

FIRST["epsilon"] = {"epsilon"}

changed = True

while changed:

changed = False

for A, alpha in grammar:

if not alpha:

if "epsilon" not in FIRST[A]:

FIRST[A].add("epsilon"); changed = True

else:

for X in alpha:

before = len(FIRST[A])

FIRST[A] |= (FIRST[X] - {"epsilon"})

if "epsilon" in FIRST[X]:

continue

break

else:

if "epsilon" not in FIRST[A]:

FIRST[A].add("epsilon"); changed = True

# FOLLOW

FOLLOW = {A: set() for A in nonterminals}

FOLLOW["Program'"].add("$")

changed = True

while changed:

changed = False

for A, alpha in grammar:

trailer = FOLLOW[A].copy()

for X in reversed(alpha):

if X in nonterminals:

before = len(FOLLOW[X])

FOLLOW[X] |= trailer

if len(FOLLOW[X]) != before:

changed = True

if "epsilon" in FIRST[X]:

trailer |= (FIRST[X] - {"epsilon"})

else:

trailer = FIRST[X] - {"epsilon"}

else:

trailer = {X}

# LR(0) 아이템·closure·goto 정의

def closure(items):

C = set(items)

while True:

new = set()

for A, alpha, dot in C:

if dot < len(alpha):

B = alpha[dot]

if B in nonterminals:

for lhs, rhs in grammar:

if lhs == B:

new.add((B, tuple(rhs), 0))

if new <= C:

break

C |= new

return C

def goto(I, X):

moved = {(A, alpha, dot+1) for A, alpha, dot in I if dot < len(alpha) and alpha[dot] == X}

return closure(moved)

# 상태 생성

states = []

init = closure({("Program'", tuple(grammar[0][1]), 0)})

states.append(init)

changed = True

while changed:

changed = False

for I in states:

for X in nonterminals | terminals:

J = goto(I, X)

if J and J not in states:

states.append(J)

changed = True

# 상태별 인덱스

state_ids = {frozenset(s): i for i, s in enumerate(states)}

# ACTION / GOTO 테이블 생성

action_table = {}

goto_table   = {}

prod_index = { (lhs, tuple(rhs)): i for i, (lhs, rhs) in enumerate(grammar) }

for I in states:

i = state_ids[frozenset(I)]

for A, alpha, dot in I:

# shift

if dot < len(alpha):

a = alpha[dot]

if a in terminals:

j = state_ids[frozenset(goto(I, a))]

action_table[(i, a)] = ('shift', j)

else:

# accept

if A == "Program'":

action_table[(i, '$')] = ('accept', None)

else:

# reduce

idx = prod_index[(A, alpha)]

for a in FOLLOW[A]:

action_table[(i, a)] = ('reduce', idx)

# GOTO 채우기

for A in nonterminals:

J = goto(I, A)

if J:

j = state_ids[frozenset(J)]

goto_table[(i, A)] = j

SLR_TABLE = {} # Dict[int, Dict[str, Union[str,int]]]

# 1) ACTION 항목: shift/reduce/accept

for (state, term), (kind, val) in action_table.items():

entry = None

if kind == 'shift':

entry = f"s{val}"

elif kind == 'reduce':

entry = f"r{val}"

elif kind == 'accept':

entry = "acc"

SLR_TABLE.setdefault(state, {})[term] = entry

# 2) GOTO 항목: next state 번호 (int)

for (state, nonterm), dest in goto_table.items():

SLR_TABLE.setdefault(state, {})[nonterm] = dest

return SLR_TABLE

given_grammar = [

("Program'", ["Program"]),

("Program", ["DeclList"]),

("DeclList", ["Decl", "DeclList"]),

("DeclList", ["epsilon"]),

("Decl", ["VarDecl"]),

("Decl", ["FuncDecl"]),

("VarDecl", ["type", "id", ";"]),

("VarDecl", ["type", "id", "=", "Expr", ";"]),

("FuncDecl", ["type", "id", "(", "ParamList", ")", "Block"]),

("ParamList", ["Param", ",", "ParamList"]),

("ParamList", ["Param"]),

("ParamList", ["epsilon"]),

("Param", ["type", "id"]),

("Block", ["{", "StmtList", "}"]),

("StmtList", ["Stmt", "StmtList"]),

("StmtList", ["epsilon"]),

("Stmt", ["IfStmt"]),

("Stmt", ["OtherStmt"]),

("IfStmt", ["if", "(", "Expr", ")", "then", "Block", "ElseStmt"]),

("ElseStmt", ["else", "Block"]),

("ElseStmt", ["epsilon"]),

("OtherStmt", ["while", "(", "Expr", ")", "Stmt"]),

("OtherStmt", ["for", "(", "Expr", ";", "Expr", ";", "Expr", ")", "Stmt"]),

("OtherStmt", ["return", "Expr", ";"]),

("OtherStmt", ["VarDecl"]),

("OtherStmt", ["ExprStmt"]),

("OtherStmt", ["Block"]),

("ExprStmt", ["id", "=", "Expr", ";"]),

("Expr", ["EqualExpr"]),

("EqualExpr", ["EqualExpr", "==", "AddExpr"]),

("EqualExpr", ["AddExpr"]),

("AddExpr", ["AddExpr", "+", "MulExpr"]),

("AddExpr", ["MulExpr"]),

("MulExpr", ["MulExpr", "*", "UnaryExpr"]),

("MulExpr", ["UnaryExpr"]),

("UnaryExpr", ["-", "UnaryExpr"]),

("UnaryExpr", ["OtherExpr"]),

("OtherExpr", ["id", "(", "ArgList", ")"]),

("OtherExpr", ["id"]),

("OtherExpr", ["num"]),

("OtherExpr", ["(", "Expr", ")"]),

("ArgList", ["Expr", ",", "ArgList"]),

("ArgList", ["Expr"]),

("ArgList", ["epsilon"]),

]

def preprocess_read_tokens(data):

# 합침

content = ' '.join(data) if isinstance(data, list) else data

# '==' 을 우선 한 덩어리로

content = re.sub(r'==', ' == ', content)

# 실제 문법에서 쓰는 기호들 띄어쓰기

# '=' 은 단일 등호만 아래 정규식으로 처리

for ch in ['+', '*', '-', '(', ')', '{', '}', ',', ';']:

content = content.replace(ch, f' {ch} ')

# '=' 토큰 분리

content = re.sub(r'(?<![=])=(?![=])', ' = ', content)

return [tok for tok in content.split() if tok]

3-2. 결과 예시

예시 1 :

type id ( type id , type id ) {

type id = id ( id , num ) ;

while ( id == num ) {

id = id + num ;

}

return id ;

}

결과 :

Accepted

Program

DeclList

Decl

FuncDecl

type

id

(

ParamList

Param

type

id

,

ParamList

Param

type

id

)

Block

{

StmtList

Stmt

OtherStmt

VarDecl

type

id

=

Expr

EqualExpr

AddExpr

MulExpr

UnaryExpr

OtherExpr

id

(

ArgList

Expr

EqualExpr

AddExpr

MulExpr

UnaryExpr

OtherExpr

id

,

ArgList

Expr

EqualExpr

AddExpr

MulExpr

UnaryExpr

OtherExpr

num

)

;

StmtList

Stmt

OtherStmt

while

(

Expr

EqualExpr

EqualExpr

AddExpr

MulExpr

UnaryExpr

OtherExpr

id

==

AddExpr

MulExpr

UnaryExpr

OtherExpr

num

)

Stmt

OtherStmt

Block

{

StmtList

Stmt

OtherStmt

ExprStmt

id

=

Expr

EqualExpr

AddExpr

AddExpr

MulExpr

UnaryExpr

OtherExpr

id

+

MulExpr

UnaryExpr

OtherExpr

num

;

StmtList

}

StmtList

Stmt

OtherStmt

return

Expr

EqualExpr

AddExpr

MulExpr

UnaryExpr

OtherExpr

id

;

StmtList

}

DeclList

예제 2:

type id () {

if ( num == num ) then {

id = id * id + (-id);

} else {

type id ;

}

}

결과 :

Accepted

Program

DeclList

Decl

FuncDecl

type

id

(

ParamList

)

Block

{

StmtList

Stmt

IfStmt

if

(

Expr

EqualExpr

EqualExpr

AddExpr

MulExpr

UnaryExpr

OtherExpr

num

==

AddExpr

MulExpr

UnaryExpr

OtherExpr

num

)

then

Block

{

StmtList

Stmt

OtherStmt

ExprStmt

id

=

Expr

EqualExpr

AddExpr

AddExpr

MulExpr

MulExpr

UnaryExpr

OtherExpr

id

*

UnaryExpr

OtherExpr

id

+

MulExpr

UnaryExpr

OtherExpr

(

Expr

EqualExpr

AddExpr

MulExpr

UnaryExpr

-

UnaryExpr

OtherExpr

id

)

;

StmtList

}

ElseStmt

else

Block

{

StmtList

Stmt

OtherStmt

VarDecl

type

id

;

StmtList

}

StmtList

}

DeclList

예제 3 : (if문에서 Block이 아닌 경우)

type id () {

if ( num == num ) then

id = id * id + (-id);

else

type id ;

}

결과 :

Error at token 12: unexpected token 'id', expected one of ['{', 'Block']

예제 4 : (토큰이 아닌 문자)

type id ( ) {

id = id & num;

}

결과 :

Error at token 8: unexpected token '&', expected one of ['(', ')', ';', ',', '*', '+', '==']

예제 5 : (콤마 빼기)

type id(type id type id) {

return id;

}

결과 :

Error at token 5: unexpected token 'type', expected one of [',', ')']